### Objectives

Part 1: Configure Dynamic NAT with PAT

Part 2: Configure Static NAT

Part 3: Verify NAT Implementation

### Configure Dynamic NAT with PAT

#### Step 1: Configure traffic that will be permitted for NAT translations.

On R2, configure a standard ACL named R2NAT that uses three statements to permit, in order, the following private address spaces:192.168.10.0/24, 192.168.20.0/24, and 192.168.30.0/24.

R2(config)# ip access-list standard R2NAT

R2(config-std-nacl)# permit 192.168.10.0 0.0.0.255

R2(config-std-nacl)# permit 192.168.20.0 0.0.0.255

R2(config-std-nacl)# permit 192.168.30.0 0.0.0.255

#### Step 2: Configure a pool of addresses for NAT.

Configure R2 with a NAT pool named R2POOL that uses the first address in the 209.165.202.128/30 address space. The second address is used for static NAT later in Part 2.

R2(config)# ip nat pool R2POOL 209.165.202.129 209.165.202.129 netmask 255.255.255.252

#### Step 3: Associate the named ACL with the NAT pool and enable PAT.

R2(config)# ip nat inside source list R2NAT pool R2POOL overload

#### Step 4: Configure the NAT interfaces.

Configure R2 interfaces with the appropriate inside and outside NAT commands.

R2(config)# inte fa0/0

R2(config-if)# ip nat inside

R2(config-if)# inte s0/0/0

R2(config-if)# ip nat inside

R2(config-if)# inte s0/0/1

R2(config-if)# ip nat inside

R2(config-if)# inte s0/1/0

R2(config-if)# ip nat outside

## Part 2: Configure Static NAT

Refer to the Topology. Create a static NAT translation to map the local.pka inside address to its outside address.

R2(config)# ip nat inside source static 192.168.20.254 209.165.202.130

### Part 3: Verify NAT Implementation

#### Step 1: Access services across the Internet.

- From the web browser of PC1, or PC3, access the web page for cisco.pka.

- From the web browser for PC4, access the web page for local.pka.

#### Step 2: View NAT translations.

View the NAT translations on R2.

R2# show ip nat translations